Effects of climate and trade policies on future global energy trade networks

Jun U. Shepardab, Bas J. van Ruijvenb, Behnam Zakerib

a Duke University (Nicholas School of the Environment), 9 Circuit Drive, Durham, NC 27701 USA. Corresponding email: jun.shepard@duke.edu.

b International Institute for Applied Systems Analysis (IIASA), Schlossplatz 1, Laxenburg, Austria.

# Abstract

Climate and trade policy have important implications for energy security and global emissions. Here, we use a state-of-the-art global integrated assessment model, MESSAGEix, to examine the effects of 1) sustained energy trade tariff policies and 2) a global carbon emissions tax on the global energy trade network. Our results indicate that sustained tariff policies have marginal effects on the network, in terms of trade volumes and bilateral trade flows. While high import tariffs allow for significant reductions in carbon emissions due to reduced fossil fuel imports in the importing region, this effect does not translate to global carbon emission reductions because this policy only directly impacts one downstream link in the energy supply chain (i.e. distribution and trade). A carbon emission tax dramatically alters the trajectory of the energy trade network, by 1) reducing the size of the network and 2) forming new trade linkages that allow for a more complex network. The emissions tax also allows for significant reductions in both import-related and overall emissions. Additional tariff policies in the presence of an emissions tax have minimal effects on the network, energy security, and emissions; however, they increase the cost of the global energy system. Our results suggest that climate policies have more dramatic and lasting effects on the global energy trade network than any individual tariff policy. Furthermore, the climate policies require cooperation (i.e. reduced trade friction) to minimize global and regional energy system costs.

# Keywords

Integrated Assessment Models; Global energy modeling; Carbon taxes; International trade; Climate policy

# 1	Introduction

Large-scale energy transitions are required to meet the commitments put forth in the Paris Climate Agreement [1,2]. Apart from global assessments of the emission reductions, detailed analysis of the feasibility and impact of these transitions are often investigated by focusing individual regions [2–8]. This makes sense, as the energy system requirements of one region do not translate to another. However, a region’s energy system does not only rely on its own production and consumption of energy, but also the ways in which it interacts with others. These interconnections form a robust trade network [9,10], and the dynamics of this network (i.e. the rate at which trade interconnections form, grow, constrain, and break) inform the rate at which new energy technologies can be adopted.

The understanding of the global energy trade network is crucial for decision-makers who are considering climate policy. Existing literature suggests that climate policies, such as carbon emissions taxes or cap-and-trade schemes, may have a large impact on international energy markets [11]. Recent literature also suggests that the reverse is not necessarily true; trade policy may not have as big an impact on emissions reduction as previously assumed [12,13]. For this line of research, Integrated Assessment Models (IAMs) are often used to assess the complex interconnections between economic processes (e.g. demand of oil) and physical processes (e.g. carbon emissions). IAMs allow for a coherent analysis of disparate fields in environmental research, including economics, demography, atmospheric chemistry, oceanography, and ecology. Because of this wide disciplinary scope, each IAM makes explicit assumptions on how global systems operate. As such, a group of IAMs are used collectively to frame policy evaluations [14–18].

In addition to its interactions with climate policymaking, the global energy trade network also plays a key role in determining the energy security of a region. The historical approach to achieving energy security was through diversification of the trade portfolio; energy importers were (and are) considered less vulnerable when they import more types of resources from a greater number of exporters [19]. Energy independence, or the ability to meet national energy demand using only domestic resources, is also used as a key indicator for energy security [13,20]. In the last few decades, energy security indices have been updated so that they also address issues of geopolitical path dependence [21,22], energy shocks (e.g. the oil shock of the 1970’s), and the integration of new economies into the global market [23]. More recently, studies have incorporated metrics from complex network analysis (CNA) to assess the vulnerability of regional energy systems and the stability of the energy trade network overall.

Despite advances in both energy security research and IAMs, the capacity to explicitly include changes in trade networks in future projections are limited, with a few exceptions [24]. This study bridges this gap by introducing a new framework for modeling bilateral trade in MESSAGEix, one of several key global energy models [15,25], and the core of the MESSAGEix-GLOBIOM integrated assessment framework. MESSAGEix is an optimization-based model, in which the objective function is to minimize global energy system costs. This minimization mimics decision-making by each region. The most disaggregated specification to date represents the world in 14 regions. We use this specification to explicitly model trade flows between regions, which allows us to represent global energy trade as a network. We then use this framework to answer two key questions for the nexus of climate and trade policymaking.

We first analyze the emissions and energy security consequences of trade policy. Here, we focus on tariff policies and model three scenarios: a baseline tariff scenario in which rates are held at the historic median by importer, a low tariff scenario in which rates are removed by 2030, and a high tariff scenario in which historic median rates are increased by a factor of five by 2030. This reflects the upper bound on regional fluctuations in historic tariff rates. We estimate the effect of tariff scenarios on carbon emissions and examine whether these policies, taken independently, can reduce or increase global and regional emissions. We also explore the effects of these policies on energy security by using CNA metrics to assess the stability of the trade network and measuring the diversity and dependence of each region’s energy trade portfolio. Note that throughout the paper, we will use the term “trade” to mean the trade of energy resources.

Second, we analyze the trade and energy security consequences of climate policy. In this study we represent climate policies through a global tax on greenhouse gas emissions. Carbon emissions taxes are a widely discussed tool for emissions reduction and their potential consequences have been extensively studied using general equilibrium models [11,26], statistical analysis [27,28], and IAMs. We build on this literature by using MESSAGEix to model the effect of a global $27/tCO2 ($100/tC) emissions tax in 2020 that increases 5% each year. We model the effect of this tax on the global energy trade network to 2050. We then apply the same metrics as in the first question to assess global emissions, network stability, and regional energy security. Further, we model the low/high tariff scenarios under a carbon emissions tax to examine the marginal effect of trade policy in the presence of a strong climate policy. For the rest of this paper, we use the term emissions tax interchangeably with a carbon emissions tax.

We begin this paper by introducing the data used to support the bilateral trade framework in MESSAGEix and our scenario analysis. We then present the methodology for this study, including a brief introduction to MESSAGEix, the conceptual framework for the bilateral trade representation, additional developments to MESSAGEix required to support this representation, and scenario design. Finally, we discuss the results of the model and their implications for energy security and climate change mitigation.

# 2	Methodology

## 2.1	Configuring bilateral trade in MESSAGEix

MESSAGEix is a dynamic linear least-cost optimization model that is developed and maintained at the International Institute for Applied Systems Analysis (IIASA) for the purpose of modeling energy system decision-making and their associated environmental impacts. The model has been developed over the last four decades and is linked to a stylized macroeconomic model (MACRO) [15]. The model has been used extensively in international climate policy, including in the Assessment Reports published by the Intergovernmental Panel on Climate Change (IPCC) [29].

MESSAGEix is a very recent, open source iteration of the MESSAGE model. MESSAGEix provides a flexible model structure; researchers can apply their own data platform to implement the model at any geographic level. This study uses MESSAGEix in combination with the ixmp platform, a database that includes historical data on global energy use patterns, shared socioeconomic pathways, and associated parameter assumptions.

MESSAGEix currently represents the world through 14 regions. A table of represented countries and their corresponding region can be found in the Supplementary Material. Trade, with the exception of gas interconnections in Europe, is represented through a global pool framework.  In this framework, regions with excess energy resources can export to a global pool from which regions that demand that resource can import. Figure 1(a) below illustrates the global pool schema as a map, while Figure 1(b) represents the schema as a flow of energy.

Fig. 1. Schematics of the MESSAGEix global energy model. (a) Illustration of the “global pool schema” representation of international trade. There are 14 representative regions. Each region can export to (solid line) or import from (dashed line) a global pool of each energy commodity (e.g. crude oil). (b) Energy flow in the global schema through inputs and output. Model “levels” are in purple (e.g. primary, secondary, export, final), nodes are in blue (i.e. regions), commodities are in green (e.g. coal, crude oil), technologies are in orange (e.g. exports, imports), and parameters are in black (e.g. input, output). AFR represents Africa, GLB represents the global node, and WEU represents Western Europe. This flow therefore represents crude oil exports from Africa to Western Europe. (c) Illustration of the “bilateral trade schema” for international trade. We explicitly model bilateral trade flows for each energy commodity, thus representing trade as a network rather than a pooled resource. (d) Flow of energy in bilateral representation of trade. Note here that technologies now explicitly define the destination of, in this instance, crude oil exports. Commodities also explicitly denote the destination (“crudeoil_weu”).

Here, commodities represent the energy resource (e.g. LNG) while the technology represents the movement of the commodity (e.g. LNG exports, LNG imports). The technology is differentiated by commodity but not by location. This is what defines the global pool schema. Note that while the global pool schema allows us to examine the total exports from/imports to regions, it does not explicitly model the trade flows among them. This explicit bilateral representation is necessary to model trade as a network and to measure security indices like trade portfolio diversity and import dependence.

The bilateral trade representation is illustrated in Figure 1(c) and 1(d). In order to explicitly delineate bilateral trade flows among regions, we needed to completely reparametrize trade in MESSAGEix. Note that in the global pool schema, the origin of fuel imports and the destination of fuel exports are not explicit in the commodity or technology. In the bilateral framework we explicitly define the destination of commodity imports (e.g. LNG_weu means LNG imports to Western Europe) and export technologies (e.g. LNG_exp_weu means LNG exports from the given region that are destined for Western Europe). This is illustrated in Figure 2(b).

For this analysis, we focus on eight commodities: coal, crude oil, ethanol, fuel oil, light oil, liquid hydrogen, LNG, and methanol. We use the BACI bilateral trade database from CEPII to extract historical bilateral trade activity at the region level for each commodity [30]. BACI includes bilateral trade flows of products in both monetary units ($1000) and weight (tons). The data are identified at the importer-exporter-product-year level, and span from 1995-2015.

We link the BACI database with the Net Calorific Value (NCV) dataset from the International Energy Agency (IEA), which includes data on the energy content (in kJ/kg) of energy resources at the country-year level.  We use the NCV data to convert units on bilateral trade flows from weight to energy. We then validate these data with the World Energy Balances (WEB) database from the IEA. The WEB has total imports and exports of energy resources from individual countries but does not represent these amounts bilaterally.

Trade under the global pool schema utilized 22 parameters. Parameters are user-defined values that build constraints and costs for the optimization model. One example of a parameter is emission factor, which defines the per unit emission by an energy technology (e.g. coal power plants). Another one is upper bound on the growth constraint for activity; this parameter sets an upper limit for how much more a technology can be adopted compared to the previous time period. This constraint ensures that the model does not allow a technology to penetrate unrealistically quickly between two time periods. A table of the parameters used in the global schema are presented in the Supplementary Material. The bilateral trade framework takes each of these parameters and applies them to each trade technology, now differentiated by region.

A key assumption underlying the global pool schema was that the characteristics, and therefore the costs, of maritime shipping are equal across energy commodities. This is clearly not a realistic assumption; for instance, the vessels that carry LNG are vastly different from the vessels that carry coal. To address the heterogeneity across shipping capacity and to ensure that trade growth does not outpace the growth of the global capacity for maritime shipping, we include additional constraints for fuel shipping. Details on these constraints can be found in the Supplementary Material.

## 2.2	The effect of sea distance on trade cost

In the bilateral representation of energy trade, we assume that, all else equal and in the absence of any sanctions or other constraints, exporters are more likely to trade with regions that are closer to them. This is based on the gravity model of bilateral trade, which assumes that trade flows are a function of the size of each economy and the distance between them [31]. To represent the trade friction from distance, we therefore need to estimate the marginal effect of distance on trade cost. Because the objective function in MESSAGEix is to minimize cost, by incorporating trade frictions through the exporting region’s variable cost, we are able to capture the impedances to trade flows.

We apply a regression analysis using data from the U.S. International Trade Commission (USITC) that are linked to the BACI data introduced above. The USITC data include information on importer and exporter GDP, population, and a slew of relative variables that can introduce or reduce trade friction.

We develop the distance-based variable cost term by: 1) measuring the shortest sea distances among regions, 2) aggregating these distances to the regional level (differentiating by energy commodity), 3) running a regression analysis to tease out the effect of distance on trade cost ($1000/ton), and 4) multiplying this effect by shortest sea distances to build our variable cost parameter. We provide further details on the methodology and results of this intermediary analysis in the Supplementary Material.

## 2.3	Scenario assumptions

We build 6 scenarios to compare the implications of energy trade and climate policies to the global energy trade network:

- Baseline tariffs (Scenario 1)

- High tariffs (Scenario 2)

- Low/no tariffs (Scenario 3)

- Emissions tax ($27/tCO2) and baseline tariffs (Scenario 4)

- Emissions tax ($27/tCO2) and high tariffs (Scenario 5)

- Emissions tax ($27/tCO2) and low tariffs (Scenario 6)

Scenarios 5 and 6 allow us to examine the marginal effect of a trade policy when an emissions tax has been imposed. We will refer to these scenario numbers for the remainder of this paper.

We derive historic tariff rates from a database of product-level ad valorem equivalent rates (AVE) from the World Trade Organization. This database includes AVE at the country-product-year level. These product codes are the same as those used in the BACI database. Tariff data are at the importer-product level, where products are identified by HS6 (Harmonized System, 6 digit) codes. A correspondence table linking HS6 codes to MESSAGEix-represented energy commodities can be found in the Supplementary Material. We follow methods from Guimbard et al. (2012) and Bouet et al. (2008) to aggregate tariffs while addressing the endogeneity between tariff rates and trade flow; details can also be found in the Supplementary Material [32,33].

In the baseline scenario, tariffs are held at the historic regional median. In the high tariff scenario, we allow tariff rates to increase to 500% of the historic regional median by 2030, after which it is held constant. This presents an important limitation in this study, namely that if a region has low tariffs historically, it will also have relatively low tariffs even after being multiplied by a factor of five. In the low tariff scenario, we allow tariff rates to reduce to 0% of the historical median by 2030.

We build the emissions tax scenario by setting the tax to the equivalent of $27/tCO2 (or $100/tC), increasing 5% annually. Table 2 displays the emissions tax at each stage of the model horizon.

| Model year | Emissions tax |
| --- | --- |
| 2020 | $100/tC |
| 2025 | $127.63/tC |
| 2030 | $162.89/tC |
| 2035 | $207.89/tC |
| 2040 | $265.33/tC |
| 2045 | $338.64/tC |
| 2050 | $432.19/tC |

Table 1. Emissions tax rates, set globally, by model year. We allow the tax to start at $100/tC and increase by 5% annually.

For each emissions tax scenario, we set tariffs to the historical median (Scenario 4), 500% of the historical median (Scenario 5), or 0% of the historical median (Scenario 6).

## 2.4	Energy security metrics

Trade portfolio diversity gives a sense of how fungible trade flows are for a given region. When a region imports from a greater number of energy exporters, it is better able to navigate shocks to one of its trade connections because it has options. Alternatively, when an importer relies on a less diverse trade portfolio, it is more vulnerable to changes in its trade partnerships; this is particularly the case when the energy exporter is a region of high political/financial volatility [21]. In this study, we measure trade diversity using the Herfindahl-Hirschman Index (HHI). The HHI measures both the diversity and market concentration of a given portfolio. The index is given by the following formula:

HHIei=jsej2

When estimating the HHI of the energy exporter,  sej2 represents the squared share of exports in fuel e going from region i to region j. When estimating the HHI of the energy importer,  sej2 represents the squared share of imports in fuel e going to region i from region j. The HHI ranges between approximately 770 and 10,000. A higher HHI suggests a more concentrated portfolio, whereas a lower HHI suggests a more diverse and balanced portfolio.

## 2.5	Limitations of this study

The main limitations of this study stem from underlying assumptions for policy scenarios. The first assumption we make is that tariff rates are increased and reduced concurrently across all regions. In reality, one region may increase a tariff, which leads another to raise their tariff rates, and so on. The same holds for a global emissions tax; perhaps one region applies a tax first, which affects another region’s decision to impose a tax. Here, we assume a global carbon emission tax. Thus, we do not capture any temporal or spatial heterogeneity in this scenario setup for the application of policies in this regard.

We also assume in our underlying country-level data that all trade between countries is shipped by sea, along the shortest shipping routes. This affects our calculation of the marginal effect of distance on trade costs. We believe that this assumption is not unrealistic, however. Firstly, 90% of total trade is conducted along maritime routes [34]. Secondly because of shipping costs (e.g. fuel costs, insurance), most shipping companies will aim to take the shortest route to transport commodities.

A limitation of the MESSAGEix global energy model is its spatial aggregation. Currently, the world is represented through 14 regions. However, there are country-level (and even sub-national level) trends that influences the structure of global energy markets. We are not able to capture this heterogeneity in this study. Furthermore, some regions combine countries that have very different energy systems; for instance, Pacific OECD includes Japan (a major energy importer) and Australia (a major energy exporter). Note, however, that MESSAGEix includes constraints to restrict Japan from tapping into the abundant energy resources available in Australia. In other words, the model setup ensures that Japan and Australian energy systems operate separately at the sub-regional level.

# 3	Results and discussion

## 3.1	Effects on network size and composition

Results suggest that tariff policies do not have a significant effect on the composition or size of the global energy trade network. We find that the only effect that the tariff policies have is on global and regional energy system costs. A high tariff scenario increases the total system cost while the low tariff scenario reduces these costs.

Conversely, we find that emissions taxes significantly alter the structure and size of the network. Under an emissions tax, the energy network contracts by nearly 50%. The network also becomes more complex; there are more than 72 trade linkages in the emissions tax scenario that were not in the baseline scenario, an increase of roughly 65%. We illustrate this increased complexity through chord diagrams in Figure 2(a)-(b) and Figure 3(a)-(l). We have also included a table of all results in the Supplementary Material.

Fig. 2. The energy trade network in 2050 under the baseline scenario (a) and under an emissions tax scenario (b). This figure illustrates the network across energy commodities, (i.e. it is the sum of coal, crude oil, ethanol, fuel oil, light oil, LNG, methanol, and liquid hydrogen). Each chord represents the net export flow. The units of these chord diagrams are in EJ. Regions are represented as follows: AFR = Africa, CPA = Centrally Planned Asia, EEU = Eastern Europe, LAM = Latin America, MEA = Middle East and North Africa, NAM = North America, PAO = Pacific OECD, PAS = Pacific Asia, RUS = Russia, SAS = South Asia, WEU = Western Europe. The color of the traded volume is identical to the exporter region.

Figure 2 illustrates the total energy trade network. Each chord is colored by the exporting region and represents the net export flow. In other words, if the Middle East (MEA) exports more LNG to North America (NAM) than it imports from NAM, the difference between the MEANAM flow and NAMMEA flow will be mapped (i.e. only the remaining MEANAM flow is illustrated). The network represents the total energy trade network, so the chords represent the sum of exports across energy commodities.

Figure 2 suggests that the weight of the global energy trade network shifts from the Middle East to Latin America under an emissions tax. The figure also shows that there is increased complexity under the tax, suggesting that previously dominant flows get spliced into more (and more diverse) linkages. Commodity-specific results presented in Figure 3 support this result and suggest that the driving mechanism for the regional shift in energy exports is driven by a shift in demand from crude oil to light oil. In other words, a reduction in the amount of crude oil traded under an emissions tax scenario is largely made up for by an increase in light oil trade. We discuss the mechanisms underlying energy-specific results below.

Fig. 3. Trade networks in 2050 under the baseline scenario (left column) and emissions tax scenario (right column). (a) and (b) represent coal trade, (c) and (d) represent crude oil trade, (e) and (f) represent ethanol trade, (g) and (h) represent fuel oil trade, (i) and (j) represent light oil trade, and (k) and (l) represent LNG trade. Total trade for the given energy commodity is given in the corresponding box in EJ. (m) illustrates the comparison between the two scenarios in terms of total trade size, by energy commodity.

For coal, we observe a stark reduction in the size of the global energy trade network. This is an intuitive result; under an emissions tax, a lot less coal moves through international trade because it becomes more expensive to use (importers are assigned the emissions associated with the trade of each energy commodity). However, in addition to reducing the amount of coal traded, an emissions tax has a secondary effect of making the network far more complex. Under the baseline scenario, there are 10 trade linkages. The linkages are dominated by flows from Centrally Planned Asia and Pacific OECD (i.e. Australia) to Pacific Asia, and from Russia to Western Europe. Under an emissions tax, there are 33 trade linkages. With the exception of a large flow from Pacific OECD from Western Europe, there are no dominating linkages in the network. Of course, the size of the flow here is also relative; the Pacific OECD-Western Europe flow under an emissions tax is 1.05EJ while the Pacific OECD-Pacific Asia flow, the largest under the baseline scenario, is 6.57EJ. We find that this occurs because major coal producing regions reduce extraction activities gradually through 2050. This is because the tax rate increases over time. By 2050, there is virtually no coal production across regions and exports are based entirely on stocks from previous years.

The shift away from crude oil is coupled with a dramatic increase the global light oil trade network. Light oil is a low-density product of crude oil refining or coal liquefication. As an importer, it is more strategic to import light oil over crude oil, even when light oil is simply a secondary product of crude, because the emissions factor is significantly lower. Our results indicate that the under the emissions tax scenario, the Middle East increases its light oil production dramatically using low-yield existing refineries to compensate for the reduction in crude oil demand overseas. This is combined with a dramatic increase in the use of light oil synthesis with carbon capture and sequestration (CCS). On the other hand, Latin America gradually increases its light oil production by increasingly investing in new higher-yield refineries and CCS. In MESSAGEix, these higher-yield refineries are modeled to have a light oil yield that is 50% greater than that of existing refinery technologies. Over time, Latin America eclipses the Middle East as the dominant light oil exporting region.

The size of the LNG trade network contracts significantly under the emissions tax scenario. At the same time, the major exporters shift from Russia and the Middle East to North America and Latin America. Unlike crude oil, which exhibits a marked reduction in trade, LNG simply does not “take off” under an emissions tax scenario. Its import demand, while increasing slightly in the short term, remains relatively low. The demand for ethanol imports increases slightly over time under the emissions tax scenario, while the demand for fuel oil declines.

For all energy commodities, the trade network becomes more complex under an emissions tax scenario. This is largely driven by the direction and magnitude of the policy effect. An emissions tax impacts all parts of the energy supply chain but has a particularly large effect on upstream activities (i.e. production and extraction). This effect moves down the supply chain. A tariff scenario, however, impacts the supply chain at the downstream (i.e. distribution). We find that the policy effect moves down the supply chain more than it moves up. This is best illustrated in the case of the Middle East. In an emissions tax scenario, it becomes prohibitively expensive to extract crude oil in the region because of the high emissions intensity of this upstream activity. This in turn impacts the amount that the Middle East exports, thus influencing the structure of the energy trade network. On the other hand, in the high tariff scenario, the effect of the policy begins with the trade network but does not extend back up to the upstream activity. We find that the extraction of oil in the high tariff scenario (76.2EJ) is not substantially different from the level of extraction in the baseline (76.7EJ).

The implications of these results are significant for the international climate policy discourse. It effectively illustrates that ambitious climate policies, such as a global emissions tax, require more interactions among trading regions. To minimize the cost of the global energy system, cooperation (i.e. reduced trade friction) will be needed. Say we use a tariff policy as a proxy for trade “friendliness” and sustained high tariffs lead to an unfriendly trade environment. Under an emissions tax, the trade network will alter regardless of the friendliness of the trade environment; however, the cost of global and regional energy systems will increase. If ambitious climate policy is undertaken, the optimal (in this case, cost-minimizing) way to prepare the trade network for such drastic shifts is to remove frictions to trade.

We validate this argument by illustrating the difference in regional energy system costs (compared to the baseline) when tariff policies are applied or removed after a carbon emissions tax is applied. We illustrate these marginal differences for major energy exporting regions (Latin America, Middle East, and Russia), major energy importing regions (Pacific Asia, South Asia, and Centrally Planned Asia), and across all regions.

Fig. 4. Average regional energy system costs (2020-2050). Major exporting regions include Latin America, Middle East, and Russia. Major importing regions include Pacific Asia, South Asia, and Centrally Planned Asia. All regions include these regions and the remaining 8 regions represented in MESSAGE. Blue lines represent regional costs under an emissions tax and high tariffs, while red lines represent regional costs under an emissions tax and reduced/no tariffs.

Figure 4 suggests that when a sustained tariff policy is applied (i.e. the trade environment is less friendly), the regional energy system costs for importing regions goes up dramatically. For energy exporting regions, the energy system costs go down initially but increase in 2050. Overall, the average system cost increases across all regions. Conversely, when energy tariffs are removed, regional energy system costs generally decline, with the exception of energy exporting regions which exhibit a slight increase in costs. Ultimately, the effect of additional energy tariff policies in the presence of an emissions tax is much larger for energy importing regions than they are for energy exporting regions. This is because energy tariffs are applied to the importer. However, the effects to importers are large enough that they dictate the direction of average regional energy system costs.

## 3.2	Effects on global emissions

For the purposes of this study, we present emissions in two ways. The first presents emissions associated with energy imports. These emissions are based on the amount and emissions factors linked to each energy commodity. In MESSAGEix, when a region exports energy, it is also exporting the emissions related to the trade of these commodities. Therefore, energy exports lead to a reduction in emissions while energy imports lead to an increase in emissions. This allows for a consumption-based accounting of regional emissions. The second presents overall emissions across all energy-related activities. It is important that we present effects on both formats of emissions because a policy that affects trade-related emissions may have minimal (or even opposite) effects on non-trade-related emissions.

Figure 5(a) presents trajectories of emissions from fuel imports, while Figure 5(b) presents global emissions trajectories. Emissions from fuel imports are the emissions associated with import activity. Each line is colored by scenario.

Fig. 5. Changes from the baseline scenario for the global level. The figure is in terms of emissions associated with fuel imports (a) and overall (b). Each line is colored by a different scenario.

Results suggest that sustained, high tariffs can reduce imported emissions by 11.2% in 2050. The low tariff scenario yields a roughly 5% increase in emissions associated with fuel imports. This, however, does not translate to an equivalent effect on overall emissions. We find that the effects of both high and low tariff policies on overall emissions trajectories are minimal. In other words, sustained tariff policies do not lead to lowered emissions and the removal of tariffs do not increase emissions overall. Our results are consistent with recent literature on the marginal effects of fossil fuel subsidies on global emissions [12].

Unsurprisingly, all emissions tax scenarios result in a large reduction in both imported and overall emissions. Additional trade policies in the presence of an emissions tax have very marginal effects on emissions trajectories. Interestingly, the high tariff scenario (without emissions tax) induces a sizeable reduction in imported emissions. However, because the high tariff scenario only affects trade-related emissions trajectories, this effect does not translate to overall emissions. The tariff only interacts with downstream costs (i.e. trade costs) in energy supply chains. As discussed in Section 3.1, a policy’s effect will be larger if it also impacts the upstream because the effect snowballs as it moves downstream. This is evidenced by the emissions tax, which significantly impacts both upstream and downstream costs. Further, the emissions tax is levied in MESSAGEix based on emissions factors assigned to each activity (e.g. extraction, liquefication, electricity generation). This means that not only is there the production-side effect of increasing extraction and distribution costs, but there is also the demand-side effect of increasing electricity generation costs. A tariff policy’s demand-side effects will only occur when energy is imported (i.e. it will cost more for importers). However, there is no additional cost once the energy has been imported for regions to use it to generate electricity. A global carbon emissions tax, because it more comprehensively impacts the energy supply chain, has a greater impact on emissions reduction, network composition, and the size of the network itself.

## 3.3	Effects on energy security

We measure energy security using the HHI introduced in the Methods section of this paper. Figure 6(a)-(b) illustrates the trajectories of exporter HHI for the two largest energy exporters (Middle East and Latin America). Figure 6(c)-(d) illustrates trajectories of the importer HHI for the two largest energy importers in the baseline scenario (Pacific Asia and South Asia). Each color represents a different scenario.

Fig. 6. Mean Herfindahl-Hirschman Index (HHI) values for major importers and exporters. The figure presents the top two energy exporting regions (a and b) and top two energy importing regions (c and d). Lines are colored by scenario.

These results indicate that neither low tariff nor high tariff scenarios influence the general magnitude or direction of both export diversity and import diversity. This is with the exception of the high tariff scenario in Pacific Asia, which exhibits reduced portfolio diversity over time when higher tariffs are levied. In general, the emissions tax scenarios allow regions to maintain a more diverse portfolio of importers. This is consistent with the effects of the emissions tax on the global fuel trade network, as increased complexity in the network is facilitated when each region has more trade linkages and balances its imports and/or exports among these linkages. Interestingly this effect does not translate generally to energy exporters; while the emissions tax allows for increased diversity in energy exports from Latin America, it actually decreases diversity in energy exports from Middle East. This is in fact a function of the dramatic reduction in crude oil demand from importing regions under the emissions tax scenario.

Our results suggest that an emissions tax can induce increased complexity in the trade network which subsequently leads to increased portfolio security. However, it is important to note here that there are also factors driving baseline trends. This study assumes SSP2 as basis for future population and GDP growth. SSP2 is the “middle-of-the-road” pathway that includes (for example) relatively high population and GDP growth in India [35,36]. We see this reflected in the energy security trajectories of this study. Figure 6 shows that, in the baseline scenario, South Asia (including India) observes increased import diversity. On the other hand, the Middle East shows a decrease in export diversity. This is driven by crude and light oil. As oil demand in South Asia increases (driven by GDP and population growth in India), it demands imports from more regions. These regions shift their oil exports to South Asia and reduce their exports to other regions. In some cases, the original trade flow to a non-South Asia importer was small to begin with, so the trade flow reduces to zero. This reduces the energy exporters’ trade diversity and increases the dependence on sustained demand from India. Over time, Latin America becomes the dominant exporter of light oil. By 2050, light oil eclipses crude oil in the global energy market. Because India increases its reliance on Latin America, we observe an increase in HHI toward 2050 which is indicative of a more concentrated import portfolio. This trajectory is not the result of any one policy, but rather the consequence of underlying socioeconomic pathways. This has implications for energy export security in the future even without any additional policy effects. This exercise underscores the importance of including underlying socioeconomic assumptions in energy policy evaluations.

## 3.4	Sensitivity analysis

We conduct two main sensitivity analyses on our results to check their robustness. The first is with regard to the tax rate in our emissions tax scenario. All scenarios assume a starting tax of $27/tCO2 in 2020, increasing 5% annually thereafter. We also test a “low emissions tax” scenario with a starting rate at $15/tCO2 and a “high emissions tax” scenario with a starting rate of $60/tCO2. We apply the 5% annual increase in both cases. By changing the tax rate, we are testing whether the existence of a tax or the underlying tax rate influences our results. In other words, if the $15/tCO2 scenario produces results that are very similar to those of the $27/tCO2 scenario, it is the existence of the tax and not the tax rate that catalyzes the dramatic changes to the global energy trade network we presented.

This result of this sensitivity analysis indicate that our findings are robust to changing tax rates, except in the case of crude oil and light oil. This is indicative of the tradeoff between the two commodities. Under the high tax scenario, crude oil trade constrains (from 39.8EJ to 27.2EJ) whereas light oil trade increases (40.5EJ to 41.5EJ). Under the low tax scenario, crude oil trade increases (to 59.1EJ) while light oil trade constrains (to 17.5EJ). This means that certain commodities are more sensitive to the emissions tax than others and that crude oil is particularly sensitive. This is consistent with the rationale presented in Section 3.1, in which we propose that the emissions tax affects costs throughout the energy supply chain and so commodities that have particularly emissions-intensive upstream costs will be disproportionately impacted by a tax.

Fig. 7. Sensitivity analysis with varying emissions tax levels. Colors represent energy commodities, shapes represent the various emissions tax scenarios. We present crude oil and light oil on a separate figure as these two commodities are traded at a much larger scale than the other commodities.

In addition to sliding the tax rate, we conduct a sensitivity analysis based on emission factors. As in the first sensitivity analysis, the purpose of this analysis was to test whether the mechanism proposed in Section 3.1 held under varying levels within each scenario. In this case, we test whether the Middle East is so heavily impacted because crude oil extraction is a very emissions-intensive activity. Our null hypothesis is that if the Middle East were to reduce its emissions factor of crude oil extraction by 50%, the global energy network under an emissions tax would more closely resemble that of the baseline scenario without the tax. In other words, the null hypothesis states that the upstream emissions factor in the Middle East is what drives the dramatic effects of the emissions tax. To run this analysis, we run the emissions tax scenarios (at $27/tCO2) while setting extraction technologies in MESSAGEix to 50% of their baseline levels.

Our results suggest that while lowering the emission factors of crude oil extraction in the Middle East has a slight effect on the composition of the global energy trade network, the presence of the emissions tax still causes dramatic changes compared to the baseline. This means that lowering the emission factor of the Middle East’s crude oil extraction is a significant, but not total, explanatory factor in determining the shape of the energy trade network. This result is consistent with the mechanism described in Section 3.1; the emissions tax has a dramatic effect on the trade network because it hits all levels of the energy supply chain. This result is also consistent with the first sensitivity analysis on tax levels.

# 4	Conclusion

Globalization is inextricably tied to energy transitions; regional energy systems are made up not only of domestic flows but also the ways in which the region interacts with others. The robust trade network that is formed by these interactions has not been explicitly modeled in widely used global energy models. This study provides a development to the MESSAGEix model by modeling the trade of energy commodities bilaterally and tracing flows of these commodities explicitly from the exporting region to the importing region. This bilateral representation allows us to closely examine effects of trade policies on emissions as well as the effects of climate policies on trade network structures. It also allows us to explore the implications of these policies on regional energy security.

Our results indicate that tariff policies, taken independently, do not significantly impact the structure of the fuel trade network. Sustained high tariffs do significantly reduce emissions that are associated with fuel imports, but do not change the trajectory of overall emissions compared to the baseline. This is also consistent with a recent study, which found that imported fuel is substituted by other domestic fossil fuel-based solutions [12]. An emissions tax of $27/tCO2 dramatically alters the fuel trade network in terms of both size (reducing it by nearly 50%) and composition of fuels. The emissions tax scenario induces large reductions in global emissions; however, these reductions are not driven by changes in trade-related emissions, but rather occur from systemic reductions throughout the energy supply chain.

Importantly, a carbon emission tax increases the complexity of the trade network by forming more trade linkages compared to the baseline. This increases the energy security of energy importers and most energy exporters, with the exception of the Middle East which relies heavily on the export of crude oil (demand for which is dramatically reduced under an emissions tax). The increased diversity of trade portfolios underscores the need for international cooperation to optimally effect a global emissions tax.

This study provides an entryway into understanding the interactions between climate and trade policy in the context of energy security and emissions. Future research on this topic should investigate the additional trade feedback stemming from existing and imminent climate change impacts to geopolitics in energy exporting regions. Another direction of future research may further explore the uncertainty of energy trajectories under climate and trade policy scenarios, particularly with regard to future demand for fossil fuels. Finally, the objective function in MESSAGE is to minimize global energy system costs; however, one can imagine that there are multiple objectives in resolving energy demand, including sustainability. We anticipate that a multi-objective optimization framework could be applied to the same questions presented in this paper.

# Funding sources

This research was supported by the National Academy of Sciences (United States). Jun U. Shepard’s doctorate is supported by the United States Office of Naval Research (Grant No. N00014-17-1-2311).

# References

[1]	United Nations. Adoption of the Paris Agreement. Conf Parties Its Twenty-First Sess 2015.

[2]	Rogelj J, Den Elzen M, Höhne N, Fransen T, Fekete H, Winkler H, et al. Paris Agreement climate proposals need a boost to keep warming well below 2 °c. Nature 2016. doi:10.1038/nature18307.

[3]	UNFCCC. INDCs as communicated by Parties. Submissions 2019.

[4]	Dong C, Dong X, Jiang Q, Dong K, Liu G. What is the probability of achieving the carbon dioxide emission targets of the Paris Agreement? Evidence from the top ten emitters. Sci Total Environ 2018. doi:10.1016/j.scitotenv.2017.12.093.

[5]	Pischke EC, Solomon B, Wellstead A, Acevedo A, Eastmond A, De Oliveira F, et al. From Kyoto to Paris: Measuring renewable energy policy regimes in Argentina, Brazil, Canada, Mexico and the United States. Energy Res Soc Sci 2019. doi:10.1016/j.erss.2018.11.010.

[6]	IEA. Energy, Climate Change & Environment - 2016 Insights. Int Energy Agency 2016.

[7]	den Elzen M, Kuramochi T, Höhne N, Cantzler J, Esmeijer K, Fekete H, et al. Are the G20 economies making enough progress to meet their NDC targets? Energy Policy 2019. doi:10.1016/j.enpol.2018.11.027.

[8]	United Nations Environment Programme, United Nations. The Emissions Gap Report 2019. 2019. doi:ISBN 978-92-807-3617-5.

[9]	Hao X, An H, Qi H, Gao X. Evolution of the exergy flow network embodied in the global fossil energy trade: Based on complex network. Appl Energy 2016. doi:10.1016/j.apenergy.2015.04.032.

[10]	Zhong W, An H, Shen L, Fang W, Gao X, Dong D. The roles of countries in the international fossil fuel trade: An emergy and network analysis. Energy Policy 2017. doi:10.1016/j.enpol.2016.07.025.

[11]	Elliott J, Foster I, Kortum S, Munson T, Cervantes FP, Weisbach D. Trade and carbon taxes. Am. Econ. Rev., 2010. doi:10.1257/aer.100.2.465.

[12]	Jewell J, McCollum D, Emmerling J, Bertram C, Gernaat DEHJ, Krey V, et al. Limited emission reductions from fuel subsidy removal except in energy-exporting regions. Nature 2018. doi:10.1038/nature25467.

[13]	Jewell J, Vinichenko V, McCollum D, Bauer N, Riahi K, Aboumahboub T, et al. Comparison and interactions between the long-term pursuit of energy independence and climate policies. Nat Energy 2016. doi:10.1038/nenergy.2016.73.

[14]	Matsuoka Y, Kainuma M, Morita T. Scenario analysis of global warming using the Asian Pacific Integrated Model (AIM). Energy Policy 1995. doi:10.1016/0301-4215(95)90160-9.

[15]	Huppmann D, Gidden M, Fricko O, Kolp P, Orthofer C, Pimmer M, et al. The MESSAGEix Integrated Assessment Model and the ix modeling platform (ixmp): An open framework for integrated and cross-cutting analysis of energy, climate, the environment, and sustainable development. Environ Model Softw 2019. doi:10.1016/j.envsoft.2018.11.012.

[16]	Stehfest E, van Vuuren D, Kram T, Bouwman L, Alkemade R, Bakkenes M, et al. Integrated assessment of global environmental change with IMAGE 3.0. 2014.

[17]	Hare B, Brecha R, Schaeffer M. Integrated Assessment Models: what are they and how do they arrive at their conclusions? Berlin: 2018.

[18]	Consortium for International Earth Science Information Network (CIESN). Thematic Guide to Integrated Assessment Modeling of Climate Change 1995.

[19]	Tergin D. Ensuring Energy Security. Foreign Aff 2006.

[20]	Greene DL. Measuring energy security: Can the United States achieve oil independence? Energy Policy 2010. doi:10.1016/j.enpol.2009.01.041.

[21]	Le Coq C, Paltseva E. Measuring the security of external energy supply in the European Union. Energy Policy 2009. doi:10.1016/j.enpol.2009.05.069.

[22]	Kruyt B, van Vuuren DP, de Vries HJM, Groenenberg H. Indicators for energy security. Energy Policy 2009. doi:10.1016/j.enpol.2009.02.006.

[23]	Yergin D. Ensuring Energy Security. Foreign Aff 2006.

[24]	Mercure JF, Pollitt H, Edwards NR, Holden PB, Chewpreecha U, Salas P, et al. Environmental impact assessment for climate change policy with the simulation-based integrated assessment model E3ME-FTT-GENIE. Energy Strateg Rev 2018. doi:10.1016/j.esr.2018.03.003.

[25]	Schrattenholzer L. The energy supply model MESSAGE. Int Inst Appl Syst Analusis, Res Report, 1981.

[26]	Bergman L. General equilibrium effects of environmental policy: A CGE-modeling approach. Environ Resour Econ 1991. doi:10.1007/BF00305950.

[27]	Rivers N, Schaufele B. The Effect of Carbon Taxes on Agricultural Trade. Can J Agric Econ Can d’agroeconomie 2015. doi:10.1111/cjag.12048.

[28]	Martin R, de Preux LB, Wagner UJ. The impact of a carbon tax on manufacturing: Evidence from microdata. J Public Econ 2014. doi:10.1016/j.jpubeco.2014.04.016.

[29]	IPCC. AR5 - Working Group 3, Mitigation of Climate Change - Contribution of Working Group III. Cambridge Univ Press 2014.

[30]	Gaulier G, Zignago S. BACI: A World Database of International Trade at the Product-level 1995-2004. 2008.

[31]	Helpman E, Melitz M, Rubinstein Y. Estimating trade flows: Trading partners and trading volumes. Q J Econ 2008. doi:10.1162/qjec.2008.123.2.441.

[32]	Guimbard H, Jean S, Mimouni M, Pichot X. MAcMap-HS6 2007, an exhaustive and consistent measure of applied protection in 2007. Econ Int 2012.

[33]	Bouët A, Decreux Y, Fontagné L, Jean S, Laborde D. Assessing applied protection across the world. Rev Int Econ 2008. doi:10.1111/j.1467-9396.2008.00753.x.

[34]	Rodrigue JP, Comtois C, Slack B. The geography of transport systems. 2016. doi:10.4324/9781315618159.

[35]	Riahi K, van Vuuren DP, Kriegler E, Edmonds J, O’Neill BC, Fujimori S, et al. The Shared Socioeconomic Pathways and their energy, land use, and greenhouse gas emissions implications: An overview. Glob Environ Chang 2017. doi:10.1016/j.gloenvcha.2016.05.009.

[36]	Fricko O, Havlik P, Rogelj J, Klimont Z, Gusti M, Johnson N, et al. The marker quantification of the Shared Socioeconomic Pathway 2: A middle-of-the-road scenario for the 21st century. Glob Environ Chang 2017. doi:10.1016/j.gloenvcha.2016.06.004.

A full description and application of the model can be found at https://message.iiasa.ac.at/en/stable/

Here we use the word “rate” interchangeably with ad valorem equivalent (AVE) rates for energy commodities

For the purposes of this study, we focus solely on the trade of physical energy resources and not on the energy that is embodied in goods and services.

Our aggregation methodology for both the BACI and NCV datasets are presented in the Supplementary Material.

We also run an ancillary scenario in which tariffs increase uniformly across regions, but do not include it in our main list of scenarios for simplicity.

We assume a minimum HHI of about 770 given that there are 13 potential trade partners for each region. If each of these partners receive an equal share of the country’s trade, we would have an HHI equal to 13(7.72), or ~771.

An ancillary scenario with uniformly increasing tariffs produces similar results, though the magnitude is different.

While the emission factor for oil extraction in the Middle East is not higher than that of other regions, most crude oil reserves are located in the Middle East. Because most extraction takes place there, changes in the emission factor would impact it most.